APPLICATION WHITE PAPER—GPON APP NOTE/EXAMPLE

Version V1.0.1

Release Date :2017-08-08

Z

Contents

DBA profile	3

SFU Authorization	3

Planning Data	3

ONU Manual authorize	4

ONU automatic authorize	6

HGU Authorization	9

Planning Data	9

ONU Manual authorize	9

ONU automatic authorize	11

Automatic Configure the ONU By Profile	14

Planning Data	14

Automatic configure onu	14

Show/Delete the ONU	21

Show the ONU status	21

Delete the ONU status	22

Case 1 SFU Bridge	23

Network Diagram	23

Planning Data	23

Configuring the OLT	24

Configuring the ONU	26

Case 2 SFU Bridge With IPTV	31

Network Diagram	31

Planning Data	31

Configuring the OLT	32

Configuring the ONU	36

Case 4 HGU Router --Internet	41

Network Diagram	41

Planning Data	41

Configuring the OLT	43

Configuring the ONU	44

Case 5 HGU Router --VoIP	53

Network Diagram	53

Planning Data	53

Configuring the OLT	54

Configuring the ONU	56

Case 6 HGU Bridge --IPTV	63

Network Diagram	63

Planning Data	63

Configuring the OLT	64

Configuring the ONU	68

# DBA profile

| Item | Description | Example |
| --- | --- | --- |
| DBA profile | The DBA of ONU | Assured :10M Maximum :100M |

By CLI   --------------

gpon-olt(config)# profile dba

gpon-olt(profile-dba:1)# type 3 assured 10000 maximum 100000

gpon-olt(profile-dba:1)# commit

By Web   --------------

# SFU Authorization

## Planning Data

| Item | Description | Example |
| --- | --- | --- |
| ONU Type | The type of onu | V2801ZG |
| ONU SN | ONU serial number | GPON00000015 |
| OLT PON | The PON port of OLT | GPON 0/1 |
| Equipid | The Equipment id of ONU | V2801ZGV8.0 |

ONU Manual authorize

By CLI   --------------

gpon-olt(config)# profile onu name V2801ZG

gpon-olt(profile-onu:3)# port-num eth 1

gpon-olt(profile-onu:3)# commit

gpon-olt(profile-onu:3)# exit

gpon-olt(config)# show onu auto-find

gpon-olt(config)# interface gpon 0/1

gpon-olt(config-pon-0/1)# onu add 1 profile V2801ZG sn GPON00000015

By Web   --------------

1、Create ONU profile for 1GE ONU

2, Find the ONU of Autofind lists and perform authorization operations on it

3, Select the 1GE ONU template

If ONU is successfully registered, working will be shown here

ONU automatic authorize

By CLI   --------------

gpon-olt(config)# profile onu name V2801ZG

gpon-olt(profile-onu:3)# port-num eth 1

gpon-olt(profile-onu:3)# commit

gpon-olt(profile-onu:3)# exit

gpon-olt(config)# show onu auto-find detail-info

gpon-olt(config)# onu auto-learn bind onu-profile V2801ZGV8.0 V2801ZG

gpon-olt(config)# interface gpon 0/1

gpon-olt(config-pon-0/1)# onu auto-learn

By Web   --------------

1、Create ONU profile for 1GE ONU

2, Equipment ID binding ONU profile

3, enable automatic learn mode of PON port

If ONU is successfully registered, working will be shown here

# HGU Authorization

## Planning Data

| Item | Description | Example |
| --- | --- | --- |
| ONU Type | The type of onu | HG326RGW |
| ONU SN | ONU serial number | GPON000010E0 |
| OLT PON | The PON port of OLT | GPON 0/1 |
| Equipid | The Equipment id of ONU | IGD |

ONU Manual authorize

By CLI   --------------

gpon-olt(config)# profile onu name HG326RGW

gpon-olt(profile-onu:3)# port-num veip 1

gpon-olt(profile-onu:3)# commit

gpon-olt(profile-onu:3)# exit

gpon-olt(config)# show onu auto-find

gpon-olt(config)# interface gpon 0/1

gpon-olt(config-pon-0/1)# onu add 1 profile HG326RGW sn GPON000010E0

By Web  --------------

1、Create ONU profile for 4GE HGU

2, Find the ONU of Autofind lists and perform authorization operations on it

3, Select the ONU template

If ONU is successfully registered, working will be shown here

ONU automatic authorize

By CLI  --------------

gpon-olt(config)# profile onu name HG326RGW

gpon-olt(profile-onu:3)# port-num veip 1

gpon-olt(profile-onu:3)# commit

gpon-olt(profile-onu:3)# exit

gpon-olt(config)# show onu auto-find detail-info

gpon-olt(config)# onu auto-learn bind onu-profile IGD HG326RGW

gpon-olt(config)# interface gpon 0/1

gpon-olt(config-pon-0/1)# onu auto-learn

By Web  --------------

1、Create ONU profile for 4GE HGU

2, Equipment ID binding ONU profile

3, enable automatic learn mode

If ONU is successfully registered, working will be shown here

# Automatic Configure the ONU By Profile

Create a line profile and srv profile. When the ONU online, it can be bound the corresponding line profile and srvprofile according to the Equipment id.

## Planning Data

| Item | Description | Example |
| --- | --- | --- |
| ONU Type | The type of onu | 2801ZG |
| ONU SN | ONU serial number | GPON000000C0 |
| OLT PON | The PON port of OLT | GPON 0/1 |
| Equipment id | The Equipment id of ONU | V2801ZGV8.0 |
| Tcont | Tcont | 1 |
| Gemport | Gemport | 1 |
| Service | The onu Service | Vlan 100 |
| Service-port | The onu service-port | Vlan 100 Mode tag |

Automatic configure onu

By CLI  --------------

gpon-olt(config)# profile onu name V2801ZG

gpon-olt(profile-onu:3)# port-num eth 1

gpon-olt(profile-onu:3)# commit

gpon-olt(profile-onu:3)# exit

gpon-olt(config)# profile dba name dba_1

gpon-olt(profile-dba:4)# type 3 assured 10000 maximum 100000

gpon-olt(profile-dba:4)# exit

gpon-olt(profile-dba:4)# commit

gpon-olt(config)# profile line name line1

gpon-olt(profile-line:1)# tcont 1 dba dba_1

gpon-olt(profile-line:1)# gemport 1 tcont 1

gpon-olt(profile-line:1)# service 1 gemport 1 vlan 100

gpon-olt(profile-line:1)# service-port 1 gemport 1 uservlan 100 vlan 100

gpon-olt(profile-line:1)# commit

gpon-olt(profile-line:1)# exit

pon-olt(config)# profile srv name srv1

gpon-olt(profile-srv:1)# portvlan eth 1 mode hybrid def_vlan 100

gpon-olt(profile-srv:1)# commit

gpon-olt(profile-srv:1)# exit

gpon-olt(config)# onu auto-learn bind onu-profile V2801ZG8.0 V2801ZG

gpon-olt(config)# onu auto-learn bind line-profile V2801ZGV8.0 line1

gpon-olt(config)# onu auto-learn bind srv-profile V2801ZGV8.0 srv1

gpon-olt(config)# interface gpon 0/1

gpon-olt(config-pon-0/1)# onu auto-learn

gpon-olt(config-pon-0/1)#exit

By Web   --------------

1、Create ONU profile for 1GE ONU

2, Equipment ID bind ONU profile

3, create line profile

4, create tcont

5, create gemport

6 create service

7、create service-port

8、create service profile

9、create service profile

10、bind profile

11, enable automatic learn mode

Show/Delete the ONU

Show the ONU status

By CLI   --------------

gpon-olt(config-pon-0/1)# show onu state

OnuIndex    Admin State    OMCC State    Phase State    Config State    Channel

---------------------------------------------------------------------------------------------------------------------------

1/1/1:1     enable         enable        working        succeeded       1(GPON)

By Web   --------------

Delete the ONU status

By CLI   --------------

gpon-olt(config-pon-0/1)# no onu 1

By Web   --------------

# Case 1 SFU Bridge

DHCP/PPPoE/Static

Network Diagram

10.1.1.1/24

## Planning Data

| Item |  | Description | Example |
| --- | --- | --- | --- |
| ONU Information | Slot No. | The number of the slot actually used. | 0 |
|  | PON No. | The number of the actually used PON port. | 1 |
|  | ONU Auth No. | Configure according to the network planning of the operator. | 1 |
|  | ONU Type | The type of the ONU. | 2801ZG |
|  | ONU SN | ONU serial number | GPON000000C0 |
|  | OLT PON | The PON port of OLT | GPON 0/1 |
|  | Equipid | The Equipment id of ONU | V2801ZGV8.0 |
| OLT | Service Uplink VLAN | The VLAN ID of the uplink port service. | VLAN mode = UNTAG PVID = 100 |
|  | Service Uplink port | Configure according to the number of the actually used uplink port. | GE8 |
| ONU | Service LAN port | The number of the actually used ONU port. | LAN1 |
|  | Tcont | The onu Tcont | Tcont=1 |
|  | Gemport | The onu Gemport | Gemport=1 |
|  | Dba | The ONU dba profile | Dba name= dba_1 Dba type = 3 |
|  | Service vlan | The vlan of ONU service. | Service Vlan = 100 |
|  | Service-port uservlan | The vlan of onu service-port | Service-port vlan =100 |
|  | Portvlan | The vlan of onu port | Portvlan=100 Mode=hybrid |
|  | If DHCP mode | The router use DHCP assign ip to PC. | Gateway = 10.1.1.1 IP Range 10.1.1.2-10.1.1.254 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |
| PC | If PPPoE mode | The router use PPPoE assign ip to PC. | IP Range 10.1.1.2-10.1.1.254 IP Mask = 255.255.255.0 User name = test Password = test |
|  | If Static mode | The router use Static assign ip to PC. | Gateway = 10.1.1.1 IP = 10.1.1.100 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |

## Configuring the OLT

By CLI   --------------

gpon-olt(config)# vlan 100

gpon-olt(config-vlan-100)# exit

gpon-olt(config)# interface gigabitethernet 0/8

gpon-olt(config-if-ge0/8)# switchport hybrid vlan 100 untagged

gpon-olt(config-if-ge0/8)# switchport hybrid pvid vlan 100

gpon-olt(config-if-ge0/8)# exit

By Web   --------------

1, Create a new VLAN

2. Configure GE port VLAN mode

3, Configure GE port PVID

## Configuring the ONU

By CLI   --------------

gpon-olt(config)#profile dba name dba_1

gpon-olt(profile-dba:1)# type 3 assured 10000 maximum 1000000

gpon-olt(profile-dba:1)# commit

gpon-olt(profile-dba:1)# exit

gpon-olt(config)# interface gpon  0/1

gpon-olt(config-pon-0/1)# onu 1 tcont 1 dba dba_1

gpon-olt(config-pon-0/1)# onu 1 gemport 1 tcont 1

gpon-olt(config-pon-0/1)# onu 1 service 1 gemport 1 vlan 100

gpon-olt(config-pon-0/1)#onu 1 service-port 1 gemport 1 uservlan 100 vlan 100

gpon-olt(config-pon-0/1)#onu 1 portvlan eth 1 mode hybrid def_vlan 100

By Web   --------------

1, create DBA profile

2,Create ONU profile for 1GE ONU

3,Authorize ONU

4, Configure ONU

Case 2 SFU Bridge With IPTV

Network Diagram

Planning Data

| Item |  | Description | Example |
| --- | --- | --- | --- |
| ONU Information | Slot No. | The number of the slot actually used. | 0 |
|  | PON No. | The number of the actually used PON port. | 1 |
|  | ONU Auth No. | Configure according to the network planning of the operator. | 1 |
|  | ONU Type | The type of the ONU. | 2801ZG |
|  | ONU SN | ONU serial number | GPON000000C0 |
|  | OLT PON | The PON port of OLT | GPON 0/1 |
|  | Equipid | The Equipment id of ONU | V2801ZGV8.0 |
|  | Service PON port | Configure according to the number of the actually used PON port. | PON1 |
| ONU | Service LAN port | The number of the actually used ONU port. | LAN1 |
|  | Tcont | The onu Tcont | Tcont=1 |
|  | Gemport | The onu Gemport | Gemport=1 |
|  | Dba | The ONU dba profile | Dba name=dba_1 Dba type = 3 |
|  | Service vlan | The vlan of ONU service. | Service Vlan = 300 |
|  | Service-port uservlan | The vlan of onu service-port | Service-port vlan =300 |
|  | Portvlan | The vlan of onu port | Portvlan=300 Mode=hybrid |
| IGMP | IGMP Group | The IP of IGMP source | IP = 239.0.0.1 |

Configuring the OLT

CLI

gpon-olt(config)# vlan 300

gpon-olt(config-vlan-300)# exit

gpon-olt(config)# interface gigabitethernet 0/7

gpon-olt(config-if-ge0/7)# switchport hybrid vlan 300 untagged

gpon-olt(config-if-ge0/7)# switchport hybrid pvid vlan 300

gpon-olt(config-if-ge0/7)# exit

gpon-olt(config)# ip igmp snooping enable

gpon-olt(config)# ip igmp snooping general-query-packet enable

gpon-olt(config)# ip igmp snooping mrouter vlan 300 interface gigabitethernet 0/7

gpon-olt(config)#ip igmp snooping mvlan 300 unknown-multicast drop igmp trap-to-cpu

gpon-olt(config)# interface gpon 0/1

gpon-olt(config-pon-0/1)# ip igmp snooping user-vlan 300 group-vlan 300 tagged

gpon-olt(config-pon-0/1)# exit

1,create a new VLAN for IGMP

2, configure VLAN mode

3,enable IGMP snooping function

Configuring the ONU

CLI

pon-olt(config)# interface gpon  0/1

gpon-olt(config-pon-0/1)# onu 1 tcont 1 dba dba_1

gpon-olt(config-pon-0/1)# onu 1 gemport 3 tcont 1

gpon-olt(config-pon-0/1)# onu 1 service 1 gemport 3 vlan 300

gpon-olt(config-pon-0/1)#onu 1 service-port 1 gemport 3 uservlan 300 vlan 300

gpon-olt(config-pon-0/1)#onu 1 portvlan eth 1 mode hybrid def_vlan 300

gpon-olt(config-pon-0/1)# onu 1 mvlan 300

gpon-olt(config-pon-0/1)# onu 1 mvlan tag-strip eth 1

gpon-olt(config-pon-0/1)# exit

create DBA profile

2, Create ONU profile for 1GE ONU

3,Authorize ONU

4,Configure ONU

Case 4 HGU Router --Internet

DHCP/PPPoE/Static

Network Diagram

10.1.1.1/24

Planning Data

| Item |  | Description | Example |
| --- | --- | --- | --- |
| ONU Information | Slot No. | The number of the slot actually used. | 0 |
|  | PON No. | The number of the actually used PON port. | 1 |
|  | ONU Auth No. | Configure according to the network planning of theoperator. | 1 |
|  | ONU Type | The type of the ONU. | HG326RGW |
| OLT | Service Uplink VLAN | The VLAN ID of the uplink port service. | VLAN mode = UNTAG PVID = 100 |
|  | Service Uplink port | Configure according to the number of the actuallyused uplink port. | GE8 |
|  | Service PON VLAN | The VLAN ID of the PON port service. | VLAN mode = TAG VLAN ID = 100 |
|  | Service PON port | Configure according to the number of the actuallyused PON port. | PON1 |
| ONU | Service LAN VLAN | The VLAN ID of the LAN port service. | Default No VLAN |
|  | Service LAN port | The number of the actually used ONU port. | LAN1 |
|  | Service LAN VLAN | The VLAN ID of the LAN port service. | VLAN mode = TAG PVID = 100 |
|  | Service LAN port | The number of the actually used ONU port. | LAN1 |
|  | Tcont | The onu Tcont | Tcont=1 |
|  | Gemport | The onu Gemport | Gemport=1 |
|  | Dba | The ONU dba profile | Dba name=dba_1 Dba type = 3 |
|  | Service vlan | The vlan of ONU service. | Service Vlan = 100 |
|  | Service-port uservlan | The vlan of onu service-port | Service-port vlan =100 |
|  | WiFi | Wireless’s SSID name,Security. | SSID1 name = SSID-1 Security method = WAP2 Mixed WPA Encryption = TKIP WPA2 Encryption = AES KEY = 12345678 |
|  | Service WAN VLAN | The VLAN ID of the PON(WAN) port service. | VLAN ID = 100 |
|  | If WAN use DHCP mode | The router use DHCP assign ip to terminal user. | Gateway = 10.1.1.1 IP Range 10.1.1.2-10.1.1.254 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |
|  | If WAN use PPPoE mode | The router use PPPoE assign ip to terminal user. | IP Range 10.1.1.2-10.1.1.254 IP Mask = 255.255.255.0 User name = test Password = test |
|  | If WAN use Static mode | The router use Static assign ip to terminal user. | Gateway = 10.1.1.1 IP = 10.1.1.100 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |
| PC | DHCP mode | The ONU use DHCP assign ip to PC. | Gateway = 192.168.1.1 IP = 192.168.1.2 IP Mask = 255.255.255.0 DNS = 192.168.1.1 |

Configuring the OLT

By CLI   --------------

gpon-olt(config)# vlan 100

gpon-olt(config-vlan-100)# exit

gpon-olt(config)# interface gigabitethernet 0/8

gpon-olt(config-if-ge0/8)# switchport hybrid vlan 100 untagged

gpon-olt(config-if-ge0/8)# switchport hybrid pvid vlan 100

gpon-olt(config-if-ge0/8)# exit

By web   --------------

1,Create a new VLAN

2. configure GE port VLAN mode

3, configure GE port PVID

Configuring the ONU

By CLI  --------------

gpon-olt(config)# interface gpon 0/1

gpon-olt(config-pon-0/1)# onu 1 tcont 1 dba dba_1

gpon-olt(config-pon-0/1)# onu 1 gemport 1 tcont 1

gpon-olt(config-pon-0/1)# onu 1 service 1 gemport 1 vlan 100

gpon-olt(config-pon-0/1)# onu 1 service-port 1 gemport 1 uservlan 100 vlan 100

gpon-olt(config-pon-0/1)# onu 1 portvlan veip 1 mode transparent

1, Create ONU profile for 4GE HGU

By web   --------------

2, Find the ONU of Autofind lists and perform authorization operations on it

3, Select the 1GE ONU template

4, Configure ONU

By ONU web  --------------

If DHCP mode

The connection status

If PPPoE mode

The connection status

If Static mode

The connection status

WiFi

Case 5 HGU Router --VoIP

DHCP/PPPoE/Static

Network Diagram

10.1.2.1/24

Planning Data

| Item |  | Description | Example |
| --- | --- | --- | --- |
| ONU Information | Slot No. | The number of the slot actually used. | 0 |
|  | PON No. | The number of the actually used PON port. | 1 |
|  | ONU Auth No. | Configure according to the network planning of theoperator. | 1 |
|  | ONU Type | The type of the ONU. | HG326RGW |
| OLT | Service Uplink VLAN | The VLAN ID of the uplink port service. | VLAN mode = UNTAG PVID = 200 |
|  | Service Uplink port | Configure according to the number of the actuallyused uplink port. | GE6 |
|  | Service PON VLAN | The VLAN ID of the PON port service. | VLAN mode = TAG VLAN ID = 200 |
|  | Service PON port | Configure according to the number of the actuallyused PON port. | PON1 |
| ONU | Service WAN VLAN | The VLAN ID of the PON(WAN) port service. | VLAN ID = 200 |
|  | Service POTS port | The number of the actually used ONU port. | POTS1 |
|  | Tcont | The onu Tcont | Tcont=1 |
|  | Gemport | The onu Gemport | Gemport=1 |
|  | Dba | The ONU dba profile | Dba name=dba_1 Dba type = 3 |
|  | Service vlan | The vlan of ONU service. | Service Vlan = 200 |
|  | Service-port uservlan | The vlan of onu service-port | Service-port vlan =200 |
|  | If WAN use DHCP mode | The router use DHCP assign ip to ONU. | Gateway = 10.1.2.1 IP Range 10.1.2.2-10.1.2.254 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |
|  | If WAN use PPPoE mode | The router use PPPoE assign ip to ONU. | IP Range 10.1.2.2-10.1.2.254 IP Mask = 255.255.255.0 User name = test1 Password = test1 |
|  | If WAN use Static mode | The router use Static assign ip to ONU. | Gateway = 10.1.2.1 IP = 10.1.2.100 IP Mask = 255.255.255.0 DNS = 8.8.8.8 |

Configuring the OLT

By CLI   --------------

gpon-olt(config)# vlan 200

gpon-olt(config-vlan-200)# exit

gpon-olt(config)# interface gigabitethernet 0/6

gpon-olt(config-if-ge0/6)# switchport hybrid vlan 200 untagged

gpon-olt(config-if-ge0/6)# switchport hybrid pvid vlan 200

gpon-olt(config-if-ge0/6)# exit

By Web  --------------

1, create a new VLAN

2.configure GE VLAN mode

Configuring the ONU

By CLI  --------------

gpon-olt(config-pon-0/1)# onu 1 tcont 1 dba dba_1

gpon-olt(config-pon-0/1)# onu 1 gemport 2 tcont 1

gpon-olt(config-pon-0/1)# onu 1 service 1 gemport 2 vlan 200

gpon-olt(config-pon-0/1)# onu 1 service-port 1 gemport 2 uservlan 200 to 200 transparent

By web --------------

Configure ONU

By ONU web  --------------

If DHCP mode

The connection status

If PPPoE mode

The connection status

If Static mode

The connection status

VoIP

Case 6 HGU Bridge --IPTV

DHCP/PPPoE/Static

Network Diagram

10.1.3.1/24

Planning Data

| Item |  | Description | Example |
| --- | --- | --- | --- |
| ONU Information | Slot No. | The number of the slot actually used. | 0 |
|  | PON No. | The number of the actually used PON port. | 1 |
|  | ONU Auth No. | Configure according to the network planning of theoperator. | 1 |
|  | ONU Type | The type of the ONU. | HG326RGW |
| OLT | Service Uplink VLAN | The VLAN ID of the uplink port service. | VLAN mode = UNTAG PVID = 300 |
|  | Service Uplink port | Configure according to the number of the actuallyused uplink port. | GE7 |
|  | Service PON VLAN | The VLAN ID of the PON port service. | VLAN mode = TAG VLAN ID = 300 |
|  | Service PON port | Configure according to the number of the actuallyused PON port. | PON1 |
| ONU | Service WAN VLAN | The VLAN ID of the PON(WAN) port service. | VLAN mode= Transparent |
|  | Service LAN VLAN | The VLAN ID of the LAN port service. | VLAN mode = UNTAG LAN VLAN ID = 300 Multicast VLAN ID = 300 |
|  | Service LAN port | The number of the actually used ONU port. | LAN4 |
|  | Tcont | The onu Tcont | Tcont=1 |
|  | Gemport | The onu Gemport | Gemport=1 |
|  | Dba | The ONU dba profile | Dba name=dba_1 Dba type = 3 |
|  | Service vlan | The vlan of ONU service. | Service Vlan = 100 |
|  | Service-port uservlan | The vlan of onu service-port | Service-port vlan =100 |
| STB | Management IP | The IP for STB. | Gateway = 10.1.3.1 IP = 10.1.3.100 IP Mask = 255.255.255.0 |

Configuring the OLT

By CLI  --------------

gpon-olt(config)# vlan 300

gpon-olt(config-vlan-100)# exit

gpon-olt(config)# interface gigabitethernet 0/7

gpon-olt(config-if-ge0/7)# switchport hybrid vlan 300 untagged

gpon-olt(config-if-ge0/7)# switchport hybrid pvid vlan 300

gpon-olt(config-if-ge0/7)# exit

gpon-olt(config)# ip igmp snooping enable

gpon-olt(config)# ip igmp snooping general-query-packet enable

gpon-olt(config)# ip igmp snooping mrouter vlan 300 interface gigabitethernet 0/7

gpon-olt(config)#ip igmp snooping mvlan 300 unknown-multicast drop igmp trap-to-cpu

gpon-olt(config)# interface gpon 0/1

gpon-olt(config-pon-0/1)# ip igmp snooping user-vlan 300 group-vlan 300 tagged

gpon-olt(config-pon-0/1)# exit

By web  --------------

1\create a new VLAN for IGMP

2, configure VLAN mode

3,enable IGMP snooping function

Configuring the ONU

By CLI  --------------

pon-olt(config)# interface gpon  0/1

gpon-olt(config-pon-0/1)# onu 1 tcont 1 dba dba_1

gpon-olt(config-pon-0/1)# onu 1 gemport 3 tcont 1

gpon-olt(config-pon-0/1)# onu 1 service 1 gemport 3 vlan 300

gpon-olt(config-pon-0/1)# onu 1 service-port 1 gemport 3 uservlan 300 to 300 transparent

gpon-olt(config-pon-0/1)# onu 1 mvlan 300

gpon-olt(config-pon-0/1)# onu 1 mvlan tag-strip eth 1

gpon-olt(config-pon-0/1)# exit

By web  --------------

Configure ONU

By ONU web  --------------